Modeling of Nonlinear Optical Activity Characteristics of Layered-Periodic Crystal Structures

E. G. Starodubtsev and O. D. Asenchik

Gomel State Technical University

October av. 48, 246746 Gomel, Belarus

Fax: + 375 - 232 - 479165; email: starodub@ggtu.belpak.gomel.by

Abstract

Analytical and numerical modeling of tensor characteristics of nonlinear optical activity for layered-periodic crystal structures (superlattices) is carried out. Calculations are executed in frames of the long wavelength approximation for electromagnetic field at neglecting of the harmonics generation and effect of the probe wave on the effective medium optical characteristics. The relations determining nonlinear optical activity, electrogyration effective tensors for the structures from bianisotropic layers are derived. The detailed analysis is made for the superlattices formed by cubic crystals of GaAs-type. Domains of parameters values at which the effective nonlinear gyrotropy characteristics exceed ones for the monocrystal components of the superlattices are ascertained.

1.	General Relations

Effects of parametric crystal optics of layered-periodic structures or superlattices (SL) are intensively investigated last time. Even in frames of the long wavelength approximation for electromagnetic field (the case of short-period SL) these structures can exhibit electrooptical, gyrotropic, magnetooptical properties which are distinguished from ones for the monocrystals forming SL [1-3]. The aim of the paper is investigation of nonlinear optical activity (NLOA) and electrogyration in short-period SL. For the description of electromagnetic properties of SL with account of NLOA one can use material equations [4]

Dq=[εqj]Ej+i[αqj]Hj,Bq=μqjHj−i[αjq]Ej,	(1)

where i2=-1, and the quantities

[εqj]=εqj+χqjkEk0+θqjklEk0El0,[αqj]=αqj+νqjkEk0+τqjklEk0El0	(2)

are the tensors of dielectric permittivity and optical activity depending on the field of the monochromatic controlling wave E0, q,j,k,l=1,2,3. In Eqs. (2) tensors χ and θ describing nonlinear dielectric properties are real and symmetric relatively to any permutation of the indexes that corresponds to non-absorbing media. The forms of characterizing NLOA real pseudotensors νand τ are determined by the crystallographic symmetry of the medium [4]. We will assume satisfying Eqs. (1) and (2) for the monocrystal layers and for the short-period SL at substitution of the corresponding material tensors for the effective tensors. According to Eqs. (1), components of magnetic permeability tensor μ are supposed to be not depending on the controlling electric field E0.

The general forms of effective tensors ε(e), χ(e), θ(e) and pseudotensor α(e) of linear optical activity were determined (see Refs. in [1,2]). Analogically to [4], let us consider NLOA in the given field approximation, at neglecting: 1) generation of harmonics in the effective medium, 2) effect of the probe wave with a low intensity on the material characteristics of SL. Moreover, the following assumptions will be used: 1) tensors ε (non-disturbed by field E0) and μ are diagonal, 2) μ>>αχ, τ>>αθ, that takes place in a wide range of parameters values [5,6].

Then the methods described in [2,3] and Eqs. (1) and (2) lead to the expressions

(μ33ε33)−1ν33j,(μ33ε33ε330)−1ν333,ε33−1ν3ij,(ε33ε330)−1ν3i3,μ33−1νi3j,

(3)

(μ33ε330)−1νi33,νijn,(ε330)−1νij3,

(μ33ε33)−1τ33ij,(μ33ε33ε330)−1τ333i,(μ33ε33ε330)−1τ33i3,(μ33ε33)−1(ε330)−2τ3333,ε33−1τ3mij,

(ε33ε330)−1τ3m3i,(ε33ε330)−1τ3mi3,ε33−1(ε330)−2τ3m33,μ33−1τm3ij,(μ33ε330)−1τm33i,              (4)

(μ33ε330)−1τm3i3,μ33−1(ε330)−2τm333,τijmn,(ε330)−1τij3m,(ε330)−1τijm3,(ε330)−2τij33,

where i,j,m,n=1,2. Here ε and ε0 are the permittivity tensors corresponding to the frequencies of the probe and controlling electric fields. At averaging of the form

A(e)=xA(1)+(1−x)A(2),	(5)

where x=d(1)/D is the relative thickness of the first layer in the SL period D, upper indexes e,1,2 here and below denote the quantities characterizing the effective medium, the first and the second crystal layers in the SL period, expressions (3,4) determine all the components of effective tensors ν(e), τ(e) describing NLOA. Expressions (3,4) are also true in the case of the constant controlling electric field E0. Then tensors ν(e) and τ(e) describe linear and quadratic electrogyration [5] in the considered media.

2. The Structures from GaAs-Type Crystals

Let us determine the form of tensor (e) in the practically important case of SL formed by cubic crystals of class 43m (GaAs-type). According to [7,8], these crystals are characterized by the following nonzero components of tensor  (the axes of the orthogonal coordinate system are parallel to the axes 4):

τ3311=τ1122=τ2233=−τ2211=−τ3322=−τ1133=λ,τ2323=τ3131=τ1212=−τ3232=−τ1313=−τ2121=λ',

(6)

τ3223=τ1331=τ2112=−τ2332=−τ3113=−τ1221=λ
          ,} {

where  ’, ” are real independent parameters. From expressions (4)-(6) we have (axis Z is perpendicular to the boundaries of the layers)

τ3311(e)=−τ3322(e)=λ/(με)>1/μ><1/ε>,

τ1331(e)=−τ2332(e)=λ} slash { \(  ital με rSup { size 8{0} }  \) >} }  over  {< {1} slash {μ> {1} slash {ε rSup { size 8{0} } >} } } } , matrix {
 {} # τ rSub { size 8{1212} }  rSup { size 8{ \( e \) } } = - τ rSub { size 8{2121} }  rSup { size 8{ \( e \) } } =λ'>, matrix {
 {} # τ rSub { size 8{2112} }  rSup { size 8{ \( e \) } } = - τ rSub { size 8{1221} }  rSup { size 8{ \( e \) } } =λ>,          (7)

τ2323(e)=−τ1313(e)=λ'/(με0)>1/μ><1/ε0>,

In Eqs. (7) and below the angular brackets denote averaging according to Eq. (5). It is seen from Eqs. (6) and (7) that the transition from monocrystals to SL is accompanied by increasing threefold the number of independent components of tensor τ. In this case the indexes of the nonzero components of  do not change. According to [7,8], the general form of this tensor in Eq. (7) has no analogues among monocrystal media. That determines the main characteristic properties of exhibition of NLOA and electrogyration in the considered SL and points to wide opportunities of creating new NLOA materials on the basis of SL.

Values of some components of (e) in Eqs. (7) can exceed the values of the analogical quantities of the monocrystals originating the SL. These components, besides ones which are equal to quantities ±<>, ±<’>, ±<”>, can be written in the form

λ(e)=λ/(ab)>1/a><1/b>,	(8)

where  and a, b are scalar parameters taking on values , ’, ” and  correspondingly. Then a domain of the parameters values, satisfying the condition (e)>, is determined by the system of inequalities

l < 1,     0 < x <1,     0      0,     (1 - )(1 - )(1 - x)> 1- l,	(9)

where l=a(1)/a(2)=b(1)/b(2).

According to system (9) an amplification of the considered induced gyrotropy properties at forming SL can take place at a small difference between the NLOA constants (l≈1) and a strong difference between the dielectric ((0)) and magnetic () constants (α≠1,β≠1) of the SL components. In particular, at nonmagnetic layers satisfying the condition (e)> is not possible for all the components of tensor (e) in Eqs. (7), besides τ2233(e)=−τ1133(e).

 = 0.5, l = 0.99                                                           x = 0.3, l = 0.99

l=0.9

Fig. 1                                                                          Fig. 2

Relations (8,9) determine also the analogical domains of parameters values for the considered SL at quadratic electrooptical and induced magnetooptical effects [2,3].

Graphs calculated on Eqs. (8) and (9) illustrate possibilities of realization of the condition (e)> at various values of the parameters.

 =  = 0.5	                                                                              =  = 0.5

(e)(2)

x

l=0.7

l=0.8

l=0.99

Fig. 3                                                                                      Fig. 4

References

[1]	I. V. Semchenko, "Gyrotropic properties of superlattices in the long wavelength approximation", Kristallografiya, vol. 35, no. 5, pp. 1047-1050, 1990.

[2]	E. G. Starodubtsev, I. V. Semchenko, and G. S. Mityurich, "Faraday effect and magnetogyration in superlattices in the long wavelength approximation," NATO ASI Series, 3. High technology, vol. 28, pp. 169-176, 1997.

[3]	E. G. Starodubtsev, "Electrooptical properties of short-period superlattices," in Proc. Bianisotropics’97, Glasgow, Great Britain, June 1997, pp. 297-300.

[4]	B. V. Bokut’ and A.N. Serdyukov, "To the theory of nonlinear optical activity," Optika i Spektroskopiya, vol. 38, no. 2, pp. 327-331, 1975.

[5]	O. G. Vlokh, Phemomena of spatial dispersion in parametric crystal optics, Lvov, 1984.

[6]	I. R. Shen, Principles of nonlinear optics, Moscow: Nauka, 1989.

[7]	L. M. Barkovskii and F. I. Fedorov, "Covariant form of the fourth rank tensors in crystals," Zhurnal Prikladnoi Spektroskopii, vol. 12, no. 4, pp. 727-734, 1970.

[8]	L. M. Barkovskii, "About pseudotensors of the fourth rank in crystals," Vestnik BGU, series I, no. 2, pp. 42-44, 1970.